Experiment No: 06								Date: 28/08/2026

Aim: Create and implement the following network topologies using simulator: Bus Topology, Ring Topology, Star Topology, Mesh Topology, Hybrid Topology & test their connectivity with Ping command.

Theory:

A Network Topology refers to the arrangement, whether physical or logical, of nodes (computers, switches, routers) and connecting cables in a computer network. The choice of topology significantly influences the network's cost, reliability, throughput, latency, ease of installation, and scalability.

Network topologies are categorized into two interrelated perspectives:

Physical Topology: The actual geometric arrangement of physical cables, network devices, and interface ports in the workspace.

Logical Topology: The actual communication pathway and protocol mechanisms through which data packets transfer across the physical medium regardless of the physical layout.

1. Bus Topology

Bus topology is a multipoint network architecture where all devices share a single common transmission medium, termed the backbone or bus cable. In modern simulation environments such as Cisco Packet Tracer, a bus is modeled using a linear cascade of switches interconnected with drop cables to individual host machines.

Architecture: A linear backbone cable connects all nodes sequentially. In legacy coaxial implementations, terminators (typically 50-ohm resistors) are required at both ends to absorb electrical signals and prevent signal reflection.

Working Principle: When a station transmits data, the electrical signal propagates in both directions along the bus. Every device receives the frame and inspects the destination MAC address. Only the node whose hardware address matches processes the frame; all other nodes discard it.

Advantages: Extremely easy to set up, requires minimal cable length compared to star or mesh, and is cost-effective for small network segments.

Disadvantages: A break or failure in the backbone cable breaks the entire network (single point of failure). Collision probability rises as more nodes transmit, and troubleshooting cable faults is difficult.

2. Star Topology

Star topology is the most widely deployed Local Area Network (LAN) topology in enterprise computing. In this architecture, each host maintains a dedicated point-to-point physical connection to a central intermediary device, such as a multiport Hub or Layer-2 Switch.

Architecture: All host devices are radially cabled to the central hub or switch via twisted-pair (UTP) cables terminating in RJ-45 connectors.

Working Principle: When a computer transmits data, it sends the frame directly to the central device. If a Hub is used (Physical Layer), it broadcasts the electrical bits out to all connected ports. If a Switch is used (Data Link Layer), it examines the destination MAC address and selectively forwards the frame only to the target recipient port.

Advantages: High fault isolation: failure of a single cable or computer does not affect the rest of the network. Easy expansion, simple centralized management, and straightforward troubleshooting.

Disadvantages: The central hub/switch represents a single point of failure; if the central device crashes, the whole network collapses. Additionally, it requires significantly more cabling than a bus topology.

3. Mesh Topology

In a Mesh Topology, every device is connected to other devices via dedicated point-to-point links. Mesh networks can be implemented as either a Full Mesh (where every node connects to every other node) or a Partial Mesh (where only critical nodes have redundant interconnections).

Link Calculation Formula: For a network consisting of N nodes in a Full Mesh, the total number of physical duplex links (L) required is given by:

L = [ N × (N - 1) ] / 2

Additionally, each node must possess (N - 1) physical network interfaces (I/O ports). For example, connecting 4 switches in full mesh requires (4 × 3) / 2 = 6 dedicated links, with each switch utilizing 3 inter-switch ports.

Working Principle: Direct dedicated links carry traffic only between the two connected nodes, eliminating traffic congestion, shared contention, and collisions.

Advantages: Highest reliability and fault tolerance: no single point of failure exists; if one link breaks, traffic instantly reroutes through alternate redundant links. Provides superior data privacy and security.

Disadvantages: Extremely expensive due to the massive amount of cabling and hardware I/O ports required. Complex installation, physical space constraints, and high maintenance overhead.

4. Ring Topology

In a Ring Topology, each computer is connected to exactly two adjacent devices, forming a continuous circular loop of data transmission. Packets travel along the ring in a predetermined direction (clockwise or counter-clockwise).

Architecture: Each station acts as an active repeater: it receives the incoming signal, regenerates the signal to avoid attenuation, and transmits it to the next adjacent workstation.

Working Principle (Token Passing): To prevent collisions, an access control mechanism known as Token Passing is traditionally utilized. A special frame called a 'token' circulates around the loop. A station wishing to transmit must capture the token, attach its data frame, and release it onto the ring. When the destination accepts the data, it sets an acknowledgment bit and the sender releases a new token.

Advantages: Equal access opportunities for all stations (deterministic performance). No data collisions occur. Easy to manage traffic flow without a central server.

Disadvantages: Unidirectional ring break disrupts the entire communication loop. Adding or removing a device temporarily brings down the network segment. Transmission latency increases proportionally with the number of hops along the ring.

5. Hybrid Topology

A Hybrid Topology is an engineered combination of two or more distinct fundamental topologies (such as Star-Bus, Star-Ring, or Cluster-Tree) integrated into a single unified communication structure. Hybrid topologies leverage the distinct operational strengths of each constituent topology while minimizing their architectural drawbacks.

Architecture: In our design (Star-Bus Hybrid), multiple centralized star clusters are interconnected through a linear backbone switch trunk line. Each star cluster accommodates independent department hosts.

Working Principle: Intra-cluster communication is handled locally through high-speed star connections. When an inter-departmental packet is transmitted, it traverses the backbone bus link to reach the destination switch cluster, which then directs it to the target host.

Advantages: Exceptional scalability and flexibility: enterprise networks can expand organically by appending new star clusters to the backbone without re-cabling existing infrastructure. Segmented fault isolation ensures local errors do not propagate.

Disadvantages: Complex design and configuration. Higher cost of implementation requiring managed network switches, multiport routers, and skilled network administrative personnel.

6. Network Connectivity Verification: The Ping Command

The Packet Internet Groper (ping) command is a core diagnostic utility at the Network Layer (Layer 3) based on the Internet Control Message Protocol (ICMP). It is used to verify end-to-end IP network connectivity between a source host and a destination node.

ICMP Echo Request (Type 8, Code 0): The initiating host sends an ICMP Echo Request encapsulated in an IP packet addressed to the target device.

ICMP Echo Reply (Type 0, Code 0): Upon receiving the valid Echo Request, the target host generates an ICMP Echo Reply back to the source.

Diagnostic Metrics: The command output reports key metrics: Round Trip Time (RTT in ms), Time-to-Live (TTL decremented by each router hop), and packet loss percentage (0% loss confirms healthy link connectivity).

Network Configuration Table (Cisco Packet Tracer Setup)

The table below documents the IP addressing scheme, subnet masks, and connected devices implemented in the Cisco Packet Tracer simulation file (Expt6.pkt):

| Topology | Device Name | IP Address | Subnet Mask | Connected Device / Interface |
| --- | --- | --- | --- | --- |
| Bus Topology | PC0 | 192.168.1.1 | 255.255.255.0 | Switch3 (FastEthernet0/1) |
| Bus Topology | PC1 | 192.168.1.2 | 255.255.255.0 | Switch0 (FastEthernet0/2) |
| Bus Topology | PC2 | 192.168.1.3 | 255.255.255.0 | Switch2 (FastEthernet0/2) |
| Bus Topology | PC3 | 192.168.1.4 | 255.255.255.0 | Switch1 (FastEthernet0/2) |
| Star Topology | PC4 | 192.168.1.5 | 255.255.255.0 | Hub0 (FastEthernet0) |
| Star Topology | PC5 | 192.168.1.6 | 255.255.255.0 | Hub0 (FastEthernet1) |
| Star Topology | PC6 | 192.168.1.7 | 255.255.255.0 | Hub0 (FastEthernet2) |
| Star Topology | PC7 | 192.168.1.8 | 255.255.255.0 | Hub0 (FastEthernet3) |
| Mesh Topology | PC8 | 192.168.1.10 | 255.255.255.0 | Switch5 (FastEthernet0/4) |
| Mesh Topology | PC9 | 192.168.1.11 | 255.255.255.0 | Switch6 (FastEthernet0/4) |
| Mesh Topology | PC10 | 192.168.1.12 | 255.255.255.0 | Switch7 (FastEthernet0/4) |
| Mesh Topology | PC11 | 192.168.1.13 | 255.255.255.0 | Switch8 (FastEthernet0/4) |
| Ring Topology | PC12 | 192.168.1.20 | 255.255.255.0 | Switch9 (FastEthernet0/3) |
| Ring Topology | PC13 | 192.168.1.21 | 255.255.255.0 | Switch10 (FastEthernet0/3) |
| Ring Topology | PC14 | 192.168.1.22 | 255.255.255.0 | Switch11 (FastEthernet0/3) |
| Ring Topology | PC15 | 192.168.1.23 | 255.255.255.0 | Switch12 (FastEthernet0/3) |
| Hybrid Topology | PC16 | 192.168.1.30 | 255.255.255.0 | Switch13 (FastEthernet0/1) |
| Hybrid Topology | PC17 | 192.168.1.31 | 255.255.255.0 | Switch13 (FastEthernet0/2) |
| Hybrid Topology | PC18 | 192.168.1.32 | 255.255.255.0 | Switch14 (FastEthernet0/1) |
| Hybrid Topology | PC19 | 192.168.1.33 | 255.255.255.0 | Switch14 (FastEthernet0/2) |

Table 1. Device Addressing & Topology Configuration Map

Simulation:

The five topologies were modeled and configured in Cisco Packet Tracer. The network canvas layouts and corresponding ping verification test results are documented below:

A. Bus Topology Implementation & Connectivity Test

Procedure: Configure PC0 through PC3 with static IP addresses 192.168.1.1 to 192.168.1.4. Interconnect the switches linearly using copper cross-over cables and attach each PC using copper straight-through cables.

[ SCREENSHOT PLACEHOLDER: Fig. 1 Bus Topology Network Diagram in Cisco Packet Tracer ] (Paste your Cisco Packet Tracer screenshot here)

Fig. 1 Bus Topology Network Diagram in Cisco Packet Tracer

[ SCREENSHOT PLACEHOLDER: Fig. 2 Ping Command Output from PC0 (192.168.1.1) to PC3 (192.168.1.4) ] (Paste your Cisco Packet Tracer screenshot here)

Fig. 2 Ping Command Output from PC0 (192.168.1.1) to PC3 (192.168.1.4)

B. Star Topology Implementation & Connectivity Test

Procedure: Place Hub0 at the center of the topology. Attach PC4 through PC7 radially to ports FastEthernet0 through FastEthernet3 using copper straight-through cables. Verify broadcast and ICMP response.

[ SCREENSHOT PLACEHOLDER: Fig. 3 Star Topology Network Diagram in Cisco Packet Tracer ] (Paste your Cisco Packet Tracer screenshot here)

Fig. 3 Star Topology Network Diagram in Cisco Packet Tracer

[ SCREENSHOT PLACEHOLDER: Fig. 4 Ping Command Output from PC4 (192.168.1.5) to PC7 (192.168.1.8) ] (Paste your Cisco Packet Tracer screenshot here)

Fig. 4 Ping Command Output from PC4 (192.168.1.5) to PC7 (192.168.1.8)

C. Mesh Topology Implementation & Connectivity Test

Procedure: Place four switches (Switch5, Switch6, Switch7, Switch8) and establish all 6 point-to-point links between every switch pair. Attach PCs PC8 through PC11 to their respective switches.

[ SCREENSHOT PLACEHOLDER: Fig. 5 Mesh Topology Network Diagram in Cisco Packet Tracer ] (Paste your Cisco Packet Tracer screenshot here)

Fig. 5 Mesh Topology Network Diagram in Cisco Packet Tracer

[ SCREENSHOT PLACEHOLDER: Fig. 6 Ping Command Output from PC8 (192.168.1.10) to PC11 (192.168.1.13) ] (Paste your Cisco Packet Tracer screenshot here)

Fig. 6 Ping Command Output from PC8 (192.168.1.10) to PC11 (192.168.1.13)

D. Ring Topology Implementation & Connectivity Test

Procedure: Connect four switches (Switch9, Switch10, Switch11, Switch12) in a closed circular loop. Connect hosts PC12 through PC15 to each respective switch port.

[ SCREENSHOT PLACEHOLDER: Fig. 7 Ring Topology Network Diagram in Cisco Packet Tracer ] (Paste your Cisco Packet Tracer screenshot here)

Fig. 7 Ring Topology Network Diagram in Cisco Packet Tracer

[ SCREENSHOT PLACEHOLDER: Fig. 8 Ping Command Output from PC12 (192.168.1.20) to PC15 (192.168.1.23) ] (Paste your Cisco Packet Tracer screenshot here)

Fig. 8 Ping Command Output from PC12 (192.168.1.20) to PC15 (192.168.1.23)

E. Hybrid Topology Implementation & Connectivity Test

Procedure: Create two distinct star clusters centered at Switch13 and Switch14, each hosting two PCs. Interconnect Switch13 and Switch14 with a cross-over backbone link to form a Star-Bus hybrid structure.

[ SCREENSHOT PLACEHOLDER: Fig. 9 Hybrid (Star-Bus) Topology Network Diagram in Cisco Packet Tracer ] (Paste your Cisco Packet Tracer screenshot here)

Fig. 9 Hybrid (Star-Bus) Topology Network Diagram in Cisco Packet Tracer

[ SCREENSHOT PLACEHOLDER: Fig. 10 Ping Command Output from PC16 (192.168.1.30) to PC19 (192.168.1.33) ] (Paste your Cisco Packet Tracer screenshot here)

Fig. 10 Ping Command Output from PC16 (192.168.1.30) to PC19 (192.168.1.33)

Pretest & Posttest:

Pretest Questions:

Q1. What is the fundamental difference between physical and logical topology?: Physical topology describes the actual geometric layout of cables, ports, and hardware nodes, whereas logical topology describes the path and protocol rules through which electrical/optical data signals flow between devices.

Q2. How many physical links are required to form a full mesh network with N devices?: A full mesh network requires L = [ N × (N - 1) ] / 2 physical duplex links, and each node requires (N - 1) I/O interface ports.

Q3. What is the primary operational role of terminators in a traditional bus topology?: Terminators absorb propagating electrical signals when they reach the physical ends of the coaxial cable, preventing signal reflection which causes packet collisions and channel corruption.

Q4. Explain the difference between a Hub and a Switch in a Star topology.: A Hub is a Layer-1 physical device that blindly broadcasts incoming frames to all ports, creating a single shared collision domain. A Switch is a Layer-2 intelligent device that inspects MAC addresses and forwards frames only to the designated recipient port, creating dedicated collision domains per port.

Q5. What protocol does the ping command utilize to test connectivity?: The ping command utilizes the Internet Control Message Protocol (ICMP) using ICMP Echo Request (Type 8) and ICMP Echo Reply (Type 0) packets encapsulated inside IP datagrams.

Posttest Questions:

Q1. Which topology provides the highest fault tolerance and why?: Full Mesh topology provides the highest fault tolerance because dedicated redundant links exist between every node pair. If any link suffers a physical break, traffic immediately traverses alternate redundant routes without packet loss or network disruption.

Q2. What occurs if the central switch in a Star topology suffers hardware failure?: The central switch acts as a single point of failure in a star topology. If it fails, all communication between all connected hosts is completely halted, though individual host NICs remain intact.

Q3. Why is Hybrid topology preferred in large-scale modern university and enterprise campuses?: Hybrid topology allows organizations to combine the cost-effectiveness of local star LANs with the high-speed backbone aggregation of bus or ring structures, offering superior scalability, modularity, and segmented fault containment.

Q4. In Ring topology, how does Token Passing prevent packet collisions?: Only the station currently in possession of the unique circulating 'token' frame has the authorization to transmit data on the shared physical loop. Once transmission completes and the destination confirms reception, the token is released for other nodes, inherently eliminating collisions.

Q5. What are the key diagnostic parameters reported by a successful ping execution?: A successful ping command outputs: (1) Bytes sent/received, (2) Round Trip Time (RTT) in milliseconds (minimum, maximum, average), (3) Time-to-Live (TTL), and (4) Packet loss percentage (0% confirms optimal connectivity).

Conclusion:

In this experiment, the five fundamental network topologies—Bus, Star, Mesh, Ring, and Hybrid—were successfully constructed and configured using the Cisco Packet Tracer simulator. Static IP addressing was assigned across all host machines within the 192.168.1.0/24 subnet. Network layer connectivity and link health were rigorously verified for each topology using the ICMP ping command, confirming zero packet loss across all intra-topology communications.